AUTOMATED PAINTING ROVER (APR)

Work Breakdown Structure

Requirements Definition Document

## 1. Task Objective

Convert the finalized use-case into a complete set of measurable engineering requirements. Each requirement must include specific, quantifiable values that the APR prototype must meet during testing and validation.

Fig. 1 — Requirement targets mapped onto the rover

## 2. Mobility Requirements

| Requirement | Target Value | Measurement Method |
| --- | --- | --- |
| Rover speed on vertical surface | 0.1 – 0.5 m/s | Encoder / Tachometer |
| Maximum load on vertical surface | ≥ 15 kg (self + payload) | Load cell test |
| Surface adhesion force (vacuum) | ≥ 2x rover weight | Pull-off test |
| Turning radius | ≤ 0.5 m | Physical measurement |
| Maximum incline angle | 90 degrees (vertical) | Inclined test surface |

## 3. Spray & Paint Quality Requirements

| Requirement | Target Value | Measurement Method |
| --- | --- | --- |
| Paint thickness accuracy | ± 0.5 mm of target | Dry film thickness gauge |
| Surface coverage per hour | ≥ 10 sq. meters / hr | Area covered / time taken |
| Paint tank capacity | ≥ 5 liters | Volume measurement |
| Spray nozzle spray width | 20 – 40 cm adjustable | Ruler / camera |
| Overspray waste target | ≤ 10% paint wastage | Paint consumed vs. area |
| Uniformity of coat | ≥ 90% uniform coverage | Camera / image analysis |

## 4. Power & Battery Requirements

| Requirement | Target Value | Notes |
| --- | --- | --- |
| Battery operating duration | ≥ 4 hours continuous | 24V/48V Li-Ion pack |
| Battery recharge time | ≤ 2 hours | Fast charge compatible |
| Low battery alert threshold | 20% battery remaining | IoT monitoring alert |
| Power to vacuum system | Continuous during operation | Critical — cannot be interrupted |

## 5. Safety & Operational Requirements

- Emergency stop trigger: within 0.5 seconds of command

- Overload protection on all motors — automatic shutoff

- Fall detection sensor: alarm if adhesion force drops below threshold

- Remote monitoring: live video and sensor feed to operator dashboard

- Operating temperature range: 5°C – 50°C

- IP54 rating minimum for dust and splash water protection

- Zero human presence required in immediate painting zone

## 6. Important Notes for Team

For each requirement listed above, the team must define specific measurable values — not vague statements. Examples:

- Speed of rover: 0.2 m/s on vertical surface (not just 'fast enough')

- Area coverage per hour: 10 sq. meters (not just 'efficient')

- Paint tank capacity: 5 liters minimum (not just 'enough for the job')

- Spraying thickness: ±0.5 mm (not just 'uniform coating')

- Battery duration: 4 hours minimum (not just 'long-lasting battery')

Every requirement must be testable and verifiable with a real measurement method during prototype evaluation.

## 7. Phase Deliverable

| Deliverable | Description |
| --- | --- |
| Engineering Requirements Document | All measurable specs with target values and test methods |
| Payload & Mobility Spec Sheet | Detailed mobility, adhesion, and weight requirements |
| Spray Quality Spec Sheet | Paint thickness, coverage rate, uniformity targets |
| Safety & Power Requirements List | Battery, emergency stop, IP rating, temperature range |

## 8. Traceability to WBS 1.1 Success Criteria

Every requirement above exists to satisfy a success criterion already agreed in WBS 1.1. Mapping them back shows the reviewer that the numbers aren't arbitrary — each one closes a specific gap.

| WBS 1.1 Success Criterion | Satisfied by (WBS 1.2 requirement) |
| --- | --- |
| Adheres and moves without slipping or falling | Adhesion ≥ 2x weight; fall-detection sensor; overload shutoff |
| Uniform coat across ≥ 90% of surface | Uniformity ≥ 90%; spray width 20–40 cm; thickness ± 0.5 mm |
| No manual intervention during operation | Continuous vacuum power; ≥ 4 hr battery; remote dashboard |
| Thickness deviation within ± 0.5 mm | Paint thickness accuracy requirement, dry film gauge test |
| Zero human exposure to fumes / height | Zero human presence in painting zone; IP54; 5–50°C range |
| IoT dashboard active throughout | Live video + sensor feed; low-battery alert at 20% |

## 9. Key System Components

A quick visual reference tying each subsystem to the requirement it needs to satisfy — useful if the panel asks "what does this actually look like on the rover."

Fig. 2 — APR key system components and the requirements each one satisfies

## 10. Open Items for the Team to Decide Before Review

A few numbers above are reasonable engineering targets but haven't been back-checked against actual component datasheets yet. Worth having answers ready if the panel asks:

- Vacuum pump selection: does a pump that gives ≥ 2x rover weight in adhesion force at 15 kg payload exist within the ₹15 lakh budget, or does this need a multi-pad array?

- Battery sizing: has the 4-hour / 24V-48V Li-Ion pack been checked against the continuous power draw of the vacuum pump + spray pump + motors together, not each system in isolation?

- Turning radius vs. track locomotion: a 0.5 m turning radius with track-based locomotion (chosen in WBS 1.1) needs a concrete steering mechanism decision — skid-steer vs. articulated joint.

- Fall-detection threshold: what adhesion-force value (in kPa or N) actually triggers the alarm? This needs a number before it's testable, not just "drops below threshold."

- IP54 vs. paint overspray: IP54 protects against splash water, but the rover will also be coated in wet paint during use — confirm this is sufficient or if a higher rating is needed for the spray zone specifically.

APR Project | Anurag University | Ref: INC25ETS084848